- Client Interface for WA_MODBUS.OCX ActiveX Control

| Name | Type | Data Type | Comments |
| --- | --- | --- | --- |
| Settings | Property | String | Set communication baud rate/parity/data bits/stop bits configuration, Example: “9600,N,8,1” Where baud rate=9600, parity=None, data bits=8, and stop bits=1 |
| ComPort | Property | Integer | 0 : COM1 1 : COM2 2 : COM3 3 : COM4 etc. |
| PortOpen | Property | Boolean | Set TRUE to Open PORT, FALSE to close it |
| DataAddr | Property | Long | Enter absolute data address of item in MODBUS data table, Range: (1-50,000) |
| NodeAddr | Property | Integer | MODBUS device Address, Range: (1-247) |
| DataElement(index%) | Property | Integer | To access data elements individually |
| DataQty | Property | Integer | Number of elements to read or write, (for data addresses 1-30,000 Range: 1-2000, for data addresses 30,001-50,000 Range:1-250) |
| Busy | Property | Boolean | Returns TRUE when in the midst of a READ or WRITE transaction |
| MsgTimeout(milliSeconds) | Property | Integer | Timeout value in milliseconds |
| GetReadData(dataArray) as Integer | Method |  | Returns value is array size. Call this method to get data after READ command issued (Action(0)), and TransactioDone() event has been fired. The passed parameter dataArray should be a raw (undimensioned) INTEGER array, it will be properly dimensioned upon return. |
| SetWriteData(dataArray) | Method |  | Pass in array of INTEGER data to be transmitted to the device, the write will be automatically invoked without requiring a call to Action(1) |
| Action(invokeIndex%) | Method |  | invokeIndex=0 initiates Read invokeIndex=1 initiates Write. |
| OpenPort(portStr as String) as BOOLEAN | Method |  | Returns TRUE if port was actually opened, typical portStr values are “COM1”, “COM2”, “ROCKET1”, etc. This Method must be used if a custom COM port card is installed, implementing non-conventional port names. |
| ClosePort as BOOLEAN | Method |  | Returns TRUE if port was actually closed. |
| TransactionDone(commResult as Integer) | Event |  | Fires after communication transaction is complete between host and MODBUS device, or a device time-out occurs. |

Application Notes:

- Ensure the COM port is closed before changing one of the following: COM port #, baud rate, parity, data bits, data bits, or timeout value.

- Typical order of use

- Configure port settings with call to Settings()

- Select and open port via one of two possible methods

- Select port number assigning value to the property ComPort()

- Open port by setting the property PortOpen = TRUE

Or

- Issue call to the method OpenPort(portName) which accepts the names for both standard and custom COM ports, and opens the port and the same time.

- Set communication address for device via property NodeAddr()

- Set address of value to be READ from or to WRITE to via call to DataAddr()

- Set number of items to READ or WRITE via property DataQty()

- If writing to device, set values to be written inidividually using .DataElement(index), or set values to write by passing in an array-full of data via .SetWriteData(intArray), where intArray is an array of integers having a size as indicated by DataQty.

- Invoke READ or WRITE to device by issuing a call to .Action(invokeIndex)

- When the MODBUS message has been sent to the device, and a response has been received from the device or the device has not reponded and therefore timed-out, the OCX event TransactionDone will be fired. An indication of whether the communication transaction was successful or an error was encountered is passed as a parameter in the event.

- READ from or WRITE to the device as many times as needed

- Close port when finished with device via call to the method ClosePort() or setting the property .PortOpen=FALSE