Modbus TCP Control Interface

| Name | Type | Data Type | Availability | Comments |
| --- | --- | --- | --- | --- |
| DataAddr | Property | Long | R/W at Design and Run Time | Modbus address of the first register to read or write. |
| DataQty | Property | Integer | R/W at Design and Run Time | Limited to the allowed number of items based on the type of operation (e.g. 125 for read registers) |
| NodeAddr | Property | Integer | R/W at Design and Run Time | Limited to values between 1 and 247 |
| HostName | Property | String | R/W at Design and Run Time | Can be either an IP Address or the Name |
| MsgTimeout | Property | Long | R/W at Design and Run Time | How long to wait for a response. |
| Busy | Function | Boolean | R/O at Run Time. Not Available at Design Time. |  |
| Status | Property | Integer | R/O at Run Time Not Available at Design Time | Contains result of the last communication attempt. (Same as the argument of the TransactionDone Event below) |
| DataElement(index%) | Property | Integer Return Value as well as an integer argument | R/W at Run Time. Not Available at Design Time. | Access to the array of data to send or that has been received |
| TransactionDone( status%) | Event | Integer Argument Result | Run Time | Occurs when a read or write is complete or has failed. Result argument indicates success/failure |
| Action( invokeIndex%) | Sub |  |  | InvokeIndex = 0 initiates a read. InvokeIndex = 1 initiates a write. |